December 15, 2009

Hello Oggie,

We have had success running this version of the MOCNESS acquisition software under Windows 2000. We have not tried Windows NT, nor do we plan on doing so. B.E.S.S.‘s software policy is that upgrades to our proprietary software which responds to new sensors (i.e. SBE 43 dissolved oxygen sensor) costs the customer nothing. We support use of the program on B.E.S.S. configured platforms only, running Windows 95 and Windows 2000 or XP operating system.

The software assumes that the mouse is either a bus mouse or a COM1 mouse, that the data will be coming from the deck unit into COM2 and that if there is a

GPS signal that it will be coming into COM4.  Once installed, the software offers the user options as to which COMport  for signal and GPS you wish to to use in the pull down menu.

The steps for installing the software are as follows: Rename the zipped file I have sent from MOCNESS.dat to MOCNESS.zip. Unzip the file. Copy the entire directory

to your hard drive designated C: (this is a must for the software has to have the software on drive C: to work).

You should have a subdirectory C:\ MOCNESS and sub-subdirectories

C:\MOCNESS\moccnfg, c:\ MOCNESS \mocdata, and c:\ MOCNESS \vbxfiles.

Once these files and the directories are installed on drive C, in a dos

window, do a CD C:\ MOCNESS \vbxfiles to change directories to \vbxfiles.

Issue the command pkunzip vbxfile.  This should result in all the vbx

files required by the program being unzipped. Do a search on your disk

for *.vbx files to see if there are any other vbx files on your disk

other than those you just unzipped.  If there are, check the dates.  If

there are vbx files with younger dates, you are best keeping those and

deleting those just unzipped (if otherwise, rename files that were

already on your disk).  Copy the remaining unique vbx files into your

C:\windows\system subdirectory. Also check to see if you have a

vbrun300.dll other than the one in the \vbxfiles subdirectory.  Again

check the date. If another exists, I think it is probably best to use the

one with the youngest (latest) date.   (Rename such files that you want

to disable rather than delete them if they were already on your disk.

That way if things don't work, you can put the system back to the way it

was.)  If you have no vbrun300.dll already on your disk, copy the one in

the \vbxfiles directory to c:\windows\system subdirectory.

Then create an icon on the windows desktop which when double clicked

with the left mouse button will start the program.  Bring up the

subdirectory having the mocness.exe file i.e. c:\ MOCNESS.  Highlight

the file with the mouse and drag it onto the desktop.  It should provide

a generic icon with a label that reads "Shortcut to Mocness.exe". This

wording can be modified.  By clicking on the icon with the right button

on the mouse and getting the properties listing, you can select the

MOCNESS icon for display on the desktop instead of the generic one. Use

browse to find the MOCNESS.ICO file in C:\ MOCNESS. VERY IMPORTANT: right click on the MOCNESS logo, select properties, click on the compatibility tab, check the box for 256 colors, and the box for Turn off advanced text services for this program.

Once  the program is running, you will have to provide the calibration data for the temperature, salinity, and pressure sensors. The manual describes how to do this.

Best regards,  Erich